SIMATS ENGINEERING

ASSESSMENT TOOL 2

Case study

COURSE NAME: Computer Networks COURSE CODE: CSA07

NAME: JESINTHA R [192511410]

COURSE OUTCOME COVERED

CO3: Analyze the performance of core network protocols such as TCP, UDP, HTTP, and  DNS under varying conditions

Assessment Tool Weightage: 40%

Case study

QUESTIONS:4 Total Marks:40

1.A multinational IT company uses a cloud-based video conferencing platform for  daily client meetings. During peak business hours, employees experience frequent  voice distortion, frozen video frames, and delayed communication. Network  monitoring reports indicate an 8% packet loss, 180 ms jitter, and fluctuating  bandwidth utilization. The IT administrator suspects that the issue is related to the  transport protocol or application-level buffering strategy.

Questions

1. Analyze whether the problem is primarily caused by the transport protocol  (TCP/UDP) or the application layer.

2. Justify your answer based on the communication requirements of real-time  multimedia applications.

3. Recommend suitable protocol-level or application-level improvements to  enhance conferencing quality.

2.A multinational software company hosts its web services across multiple  continents. Employees frequently report long delays before the application  homepage loads. Network analysis shows that the average DNS lookup time is  approximately 800 ms, significantly affecting user experience. The organization  plans to redesign its DNS infrastructure while ensuring continuous service  availability during server failures.

Questions

1. Analyze the reasons behind the high DNS resolution time.

2. Propose a suitable DNS architecture using techniques such as Anycast DNS,  DNS pre-fetching, and TTL optimization to reduce the lookup time below  50 ms.

3. Evaluate the advantages and limitations of your proposed architecture in terms  of performance, scalability, and availability.

3.A cloud service provider delivers applications to millions of users worldwide.  During promotional events, users experience intermittent delays in accessing cloud hosted services. Investigation reveals that DNS queries are taking nearly 800 ms because all requests are directed to a single primary DNS server. The company wants  a highly available DNS solution capable of handling global traffic efficiently.

Questions

4. Examine the impact of centralized DNS architecture on application  performance.

5. Design a distributed DNS solution that minimizes lookup latency while  maintaining high availability.

6. Analyze how TTL values, Anycast routing, and DNS caching influence  system performance and fault tolerance.

4.A financial institution operates an electronic stock trading platform where  thousands of buy and sell orders are submitted every second. During market opening  hours, the average order acknowledgment latency increases from 1 ms to 40 ms,  resulting in delayed trade confirmations and customer dissatisfaction. Network  engineers observe increased TCP retransmissions, longer connection establishment  times, and excessive buffering.

Questions

1. Analyze three possible TCP-related factors responsible for the latency  increase, considering flow control, Nagle's algorithm, and SYN queue  limitations.

2. Explain how each factor affects transaction processing in high-frequency  systems

3. Recommend appropriate TCP configuration changes to reduce latency and  improve transaction reliability.

RUBRICS FOR CALCULATION

| Criteria | Weightag e | Level 4 – Exemplary | Level 3 – Proficient | Level 2 – Developing | Level 1 – Beginning |
| --- | --- | --- | --- | --- | --- |
| Understanding of Case | 25 | Thorough understanding; clearly identifies all key issues | Good understanding with most issues identified | Basic understandi ng; some issues missed | Poor understanding; problem not identified correctly |
| Application of Concepts | 25 | Effectively applies relevant concepts to analyze the case deeply | Applies appropriate concepts with minor gaps | Limited application of concepts | Fails to apply relevant concepts |
| Analysis | 20 | In-depth analysis with strong reasoning and insights | Good analysis with logical reasoning | Basic analysis with limited depth | Weak or no analysis; lacks reasoning |
| Solution Design | 20 | Innovative, feasible solutions with strong justification | Logical solutions with adequate justification | Basic solutions with weak justification | Incorrect or no solution provided |
| Clarity | 10 | Clear, well structured, and easy to understand | Organized and understandable presentation | Limited clarity and structure | Poor clarity; difficult to understand |

CASE STUDY 1

Voice and Video Degradation on a Cloud Video-Conferencing Platform

Scenario Snapshot: A multinational firm's cloud video-conferencing service shows voice distortion, frozen frames, and delayed communication during peak hours, with 8% packet loss, 180 ms jitter, and fluctuating bandwidth.

## Q1. Is the problem caused by the transport protocol (TCP/UDP) or the application layer?

Primary diagnosis: The root cause is predominantly a transport-layer choice-and-configuration issue, compounded by an application-layer buffering strategy that reacts poorly to it. The symptom pattern — packet loss tolerated as distortion rather than as delay, combined with high jitter — is the signature of a session still running (or falling back to) TCP-style reliable delivery for a real-time media stream, rather than a lightweight, loss-tolerant transport.

If the platform were using UDP-based media transport (as WebRTC-class conferencing tools do via SRTP over UDP), an 8% loss rate would surface as brief, isolated glitches — a skipped video frame or a short audio click — because the receiver simply conceals the missing packet and moves on. What is instead reported is frozen frames and delayed communication, which indicates that lost packets are being retransmitted and that subsequent, already-arrived data is being held back until the missing segment is recovered. That "wait-for-retransmission" behaviour is characteristic of TCP's in-order, reliable delivery model, or of an application receive-buffer that is imitating it (a jitter buffer sized far too large, or a media pipeline that will not render a frame until it is complete).

Supporting evidence from the reported metrics

| Observed Symptom | Transport-Layer Reading | Application-Layer Reading |
| --- | --- | --- |
| Frozen video frames | Head-of-line blocking from TCP retransmission of lost segments | Jitter buffer/render pipeline waiting for a complete, in-order frame |
| Voice distortion | Congestion-control back-off starving the audio stream of bandwidth | Poor codec concealment / no forward error correction on packet loss |
| 180 ms jitter | Retransmission timers and TCP congestion window resets | Adaptive bitrate logic reacting too slowly to changing network state |
| Fluctuating bandwidth | TCP additive-increase/multiplicative-decrease sawtooth under loss | No dynamic bitrate/resolution scaling in the client application |

In short, the transport protocol sets the ceiling on how well real-time media can survive loss and jitter, and the application layer determines how gracefully the platform degrades within that ceiling. Here, both layers are implicated, but the transport-level behaviour (reliable, ordered delivery being applied to a delay-sensitive stream) is the primary driver, with an oversized or static application buffer amplifying the effect.

## Q2. Justification based on real-time multimedia communication requirements

Real-time voice and video have three requirements that are fundamentally incompatible with TCP's design philosophy:

- Bounded latency over guaranteed delivery — a video frame that arrives 300 ms late is worthless even if it eventually arrives intact; the human ear/eye needs current data, not perfect data.

- Loss tolerance — codecs such as Opus (audio) and VP8/H.264 (video) are designed to conceal occasional missing packets through interpolation, so a few lost packets are far less damaging than the delay caused by recovering them.

- Independent, timestamped delivery — each audio/video frame should be decodable on its own; forcing strict in-order delivery (as TCP does) means one lost packet stalls everything behind it, a problem known as head-of-line blocking.

TCP's reliability mechanisms — sequence numbers, acknowledgements, retransmission timeouts, and congestion-window collapse on loss — are optimised for throughput-accuracy trade-offs suited to file transfer or web pages, not for millisecond-level interactivity. At 8% loss, TCP's congestion control would repeatedly halve its sending rate, which directly explains the "fluctuating bandwidth" observation, while the receiver stalls delivery until retransmitted packets restore order, which directly explains frozen frames and the 180 ms jitter. UDP, by contrast, sends packets independently with no retransmission or ordering guarantee, letting the application decide in real time whether to conceal, skip, or interpolate — which is precisely the behaviour real-time conferencing needs.

## Q3. Recommended protocol-level and application-level improvements

Protocol-level (transport) improvements

- Migrate/confirm media transport to UDP-based RTP/SRTP (the standard for WebRTC-class conferencing) instead of any TCP or TCP-fallback media path; reserve TCP only for signalling (SDP/SIP/WebSocket control messages).

- Enable Forward Error Correction (FEC) and packet-loss-concealment codecs so isolated UDP losses are reconstructed locally instead of requested for retransmission.

- Deploy congestion control designed for real-time media, such as Google Congestion Control (GCC) or the IETF's WebRTC-oriented BBR-style algorithms, which respond to jitter/loss with rate adaptation rather than TCP's aggressive back-off.

- Mark media traffic with DSCP/QoS tags (e.g., EF — Expedited Forwarding) so routers prioritise it over best-effort traffic during peak-hour congestion.

Application-level improvements

- Replace any static jitter buffer with an adaptive jitter buffer that resizes dynamically based on measured network jitter (targeting the 180 ms currently observed) instead of a fixed, oversized window.

- Implement Scalable Video Coding (SVC) / adaptive bitrate so resolution and frame rate degrade smoothly under loss instead of freezing.

- Add simulcast or selective forwarding (SFU architecture) so the server, not each client, adapts stream quality per participant's link condition.

- Introduce client-side network diagnostics with automatic fallback (e.g., audio-only mode) when loss/jitter cross a defined threshold, preserving usable communication over a perfect-but-frozen stream.

CASE STUDY 2

Redesigning Global DNS Infrastructure for a Multinational Web Platform

Scenario Snapshot: Homepage load is delayed by an average DNS lookup time of ~800 ms across a multi-continent deployment. The organisation wants a redesigned DNS architecture that keeps service available during server failures.

## Q1. Reasons behind the high DNS resolution time

An 800 ms average lookup is roughly 40–80 times slower than a well-tuned DNS deployment (which typically resolves in 10–20 ms from a warm cache). The most probable contributing factors are:

- Geographic distance to authoritative servers — if authoritative name servers are centralised in one region, users on other continents incur full round-trip-time (RTT) latency for every uncached query (e.g., 200–300 ms RTT per hop across intercontinental links).

- Deep referral (recursive) chains — resolution passing through multiple recursive resolvers, forwarders, or a long delegation chain (root → TLD → authoritative) multiplies the number of round trips before an answer returns.

- Low or inconsistent TTL values — aggressively short TTLs force resolvers to re-query the authoritative server far more often than necessary, discarding the benefit of caching.

- No (or under-utilised) Anycast routing — with unicast authoritative servers, every query — regardless of origin — is routed to the same physical location rather than the nearest instance.

- Absence of DNS pre-fetching/caching at the edge or client — browsers and local resolvers are not proactively resolving domains before they are needed, so every session pays the full lookup cost.

- Resolver/server overload during peak periods — a single or small pool of DNS servers handling global query volume can queue requests, adding processing delay on top of network delay.

- Possible DNSSEC or misconfigured record chains — extra validation round trips or unnecessary CNAME chains add sequential lookups before final resolution.

## Q2. Proposed DNS architecture to bring lookup time below 50 ms

The target of sub-50 ms resolution requires attacking both the network-distance problem and the cache-efficiency problem simultaneously. The proposed architecture layers three techniques:

(a) Anycast DNS

Deploy authoritative name servers using Anycast, advertising the same IP address from multiple Points of Presence (PoPs) across continents (e.g., North America, Europe, Asia-Pacific, South America). BGP routing then automatically directs each user's query to the topologically nearest PoP, collapsing intercontinental RTT down to a regional or metro-level RTT of a few milliseconds.

(b) TTL optimisation

Set a tiered TTL strategy rather than one flat value: long TTLs (3,600–86,400 s) for stable records such as apex A/AAAA and MX records that rarely change, and moderate TTLs (60–300 s) only for records that genuinely need agility (e.g., load-balancer endpoints during failover). This maximises the proportion of queries answered from cache at the recursive resolver and client, which is where most of the 50 ms budget is actually won.

(c) DNS pre-fetching

Enable DNS prefetching (dns-prefetch resource hints) in the web application for domains the homepage will call next (CDN assets, API endpoints, third-party resources), so the resolution happens in parallel with earlier page-load work rather than blocking it. Pair this with pre-warmed resolver caches at edge PoPs so common queries are already cached before peak traffic arrives.

Supporting components

- A tiered resolver hierarchy: Anycast-based recursive resolvers at the edge, backed by regional authoritative clusters, minimising both the number of hops and the distance of each hop.

- Health-checked failover: each Anycast PoP is monitored; if a node fails, BGP withdraws its route and traffic is absorbed by the next-nearest healthy PoP within seconds, with no single point of failure.

- Secondary/standby authoritative providers (multi-vendor DNS, e.g., a primary Anycast provider plus a secondary from a different vendor) to protect against provider-level outages.

## Q3. Advantages and limitations of the proposed architecture

| Dimension | Advantages | Limitations |
| --- | --- | --- |
| Performance | Queries resolve at the nearest PoP, typically cutting lookup time from ~800 ms to single-digit/low-double-digit ms | Anycast performance depends on accurate, well-peered BGP routing; poor upstream peering can still route users to a distant PoP |
| Scalability | New PoPs can be added without changing the advertised IP; load is naturally distributed across nodes | Requires ongoing investment in global PoP infrastructure and BGP session management as traffic grows |
| Availability | Automatic failover at the routing layer; a failed PoP is bypassed within seconds with no client reconfiguration | Anycast failover is coarse-grained (whole-PoP), and BGP convergence delay can briefly affect a small subset of users during a route withdrawal |
| Cache efficiency | Longer, tiered TTLs plus pre-fetching drastically cut repeat-query volume and load on authoritative servers | Longer TTLs slow propagation of legitimate record changes, so failover of application endpoints must use short-TTL records specifically |
| Operational complexity | Centrally managed record set replicated to all PoPs, simplifying updates | Multi-vendor/secondary DNS adds coordination overhead (zone-transfer consistency, monitoring across providers) |

CASE STUDY 3

Eliminating Single-Point DNS Bottlenecks for a Global Cloud Provider

Scenario Snapshot: Millions of users experience intermittent delays during promotional events because every DNS query is directed to one primary DNS server, producing lookups of nearly 800 ms. A highly available, globally efficient DNS solution is required.

## Q4. Impact of centralised DNS architecture on application performance

Routing every query worldwide to a single primary DNS server creates three compounding problems, all of which are visible in this scenario:

- Latency penalty for distant users — every user, regardless of location, pays the full geographic RTT to reach the one server, so users far from that server experience the worst delays, and this penalty is paid on every cache miss.

- Query queuing under load — a single server has a finite processing capacity (connections, CPU, query throughput). During promotional-event traffic spikes, incoming queries queue up, adding processing delay on top of network delay — explaining why the issue is intermittent and load-correlated rather than constant.

- Single point of failure — if the primary server slows, is overloaded, or fails, there is no alternative path for resolution; the entire platform becomes unreachable for new sessions, not just degraded, since DNS resolution is the mandatory first step before any connection can be made.

- Cascading application impact — because DNS resolution sits in the critical path of every new connection (API calls, asset loads, microservice-to-microservice lookups in cloud-native systems), an 800 ms DNS delay is added in front of the platform's actual response time, compounding perceived slowness far beyond what the application layer alone would cause.

## Q5. Design of a distributed DNS solution minimising latency while maintaining high availability

The recommended design replaces the single primary server with a globally distributed, redundant resolution fabric:

- Anycast-based authoritative layer: the same DNS service IP is announced from multiple geographically dispersed PoPs (e.g., 6–10+ regions), so BGP naturally sends each query to the nearest healthy instance instead of one fixed server.

- Primary–secondary (multi-master) authoritative pairs within each region, with automated zone synchronisation, so a single node failure within a region does not remove that region's coverage.

- Multi-provider redundancy: run the zone on two independent Anycast DNS providers (different networks/ASNs) so a provider-wide outage does not take the domain offline — this is the standard mitigation for the single-point-of-failure risk identified above.

- Global load-aware traffic steering: combine Anycast with health checks and, where applicable, GeoDNS/latency-based routing so that during a promotional-event surge, traffic is spread across all healthy PoPs rather than concentrated on one.

- Elastic capacity per PoP: auto-scaled resolver capacity at each PoP to absorb event-driven spikes without queuing, addressing the load-correlated intermittency described in the case.

- Continuous health monitoring with automatic route withdrawal, so an unhealthy PoP is removed from Anycast advertisement within seconds and its traffic is absorbed by neighbouring PoPs.

## Q6. Influence of TTL values, Anycast routing, and DNS caching on performance and fault tolerance

| Mechanism | Effect on Performance | Effect on Fault Tolerance |
| --- | --- | --- |
| TTL values | Longer TTLs increase cache-hit rate at resolvers/clients, cutting repeated 800 ms-class lookups to near-zero on repeat visits | Longer TTLs slow the propagation of a failover change; a hybrid approach (long TTL for stable records, short TTL for endpoints that must fail over quickly) balances both goals |
| Anycast routing | Routes each query to the nearest PoP by network topology, collapsing intercontinental RTT to local/regional RTT | Removes the single-point-of-failure risk; BGP automatically reroutes around a failed or withdrawn PoP without client-side changes |
| DNS caching | Resolves the large majority of repeat queries locally (client or ISP resolver) without any network round trip at all | A well-populated cache lets end users continue reaching a service briefly even during a brief authoritative-layer disruption, buying time for failover to complete |

Used together, these three mechanisms are complementary rather than redundant: Anycast reduces the cost of the queries that must reach the authoritative layer, caching (governed by TTL) reduces how many queries need to reach it at all, and TTL tuning ensures that the reduction in query volume does not come at the cost of stale answers during a failover event.

CASE STUDY 4

TCP-Induced Latency in a High-Frequency Electronic Trading Platform

Scenario Snapshot: During market opening, average order-acknowledgment latency rises from 1 ms to 40 ms, with increased TCP retransmissions, longer connection setup times, and excessive buffering.

## Q1. Three TCP-related factors responsible for the latency increase

(a) Flow control

TCP flow control uses the receive window (rwnd) advertised by the receiver to prevent overwhelming it. Under the sudden burst of buy/sell orders at market opening, receive buffers on the trading server fill faster than the application can drain them, causing the advertised window to shrink. Once the window is small, the sender is forced to pause or send in smaller segments, directly adding waiting time to each order's path — exactly the kind of stall that turns a 1 ms acknowledgment into a multi-millisecond one.

(b) Nagle's algorithm

Nagle's algorithm delays sending small TCP segments so they can be coalesced into larger ones, reducing overhead on typical bulk traffic. Order messages, however, are small and latency-critical; if Nagle's algorithm is enabled, the stack will hold a small outgoing order segment until either enough data accumulates or a pending ACK is received, deliberately trading latency for efficiency — the opposite of what a trading system needs. Combined with delayed ACKs on the receiving side (which can wait up to ~40 ms before acknowledging), this pairing is a well-known and often-underestimated source of exactly this scale of latency spike.

(c) SYN queue limitations

Every new TCP connection begins with a three-way handshake, and incoming SYN requests are held in a fixed-size SYN backlog queue until accepted. At market opening, thousands of client sessions/reconnections arrive almost simultaneously; if the SYN queue is undersized, additional SYNs are dropped and must be retransmitted by the client after a timeout, adding hundreds of milliseconds to connection establishment for the affected sessions and increasing the retransmission counts observed by the network engineers.

## Q2. How each factor affects transaction processing in high-frequency systems

| TCP Factor | Mechanism of Impact | Effect on Trading Transactions |
| --- | --- | --- |
| Flow control | Shrinking receive window forces sender to throttle/pause mid-stream | Order and acknowledgment messages queue behind the throttle, delaying trade confirmation exactly when order volume (and its business value) is highest |
| Nagle's algorithm | Small order packets are held and batched instead of sent immediately | Introduces artificial send-side delay per order; interacting with delayed ACKs can add tens of milliseconds per round trip, compounding across thousands of orders per second |
| SYN queue limits | Overflowing backlog drops/delays new connection handshakes | New or reconnecting trading sessions experience slow or failed connection setup at the busiest moment, delaying the first order of the session and increasing retransmissions |

Collectively, these factors explain both symptoms engineers observed: increased retransmissions (SYN queue overflow and window-limited stalls triggering timeouts) and longer connection establishment (SYN backlog saturation), which together account for the jump from a 1 ms baseline to a 40 ms peak acknowledgment latency.

## Q3. Recommended TCP configuration changes

- Disable Nagle's algorithm on order/acknowledgment sockets (set TCP_NODELAY) so latency-sensitive small messages are sent immediately rather than batched.

- Disable or minimise delayed ACKs (TCP_QUICKACK, or reduce the delayed-ACK timer) on the counterpart side so disabling Nagle's algorithm is not neutralised by the receiver still waiting to acknowledge.

- Increase the TCP receive/send buffer sizes and enable window scaling so the flow-control window does not become the bottleneck under the market-open burst of orders.

- Enlarge the SYN backlog queue (increase the listen backlog and the kernel's SYN queue limit) and enable SYN cookies so legitimate connection bursts at market open are accepted rather than dropped.

- Tune retransmission timeout (RTO) parameters and enable TCP Fast Open where supported, to shorten the recovery path for any packet loss that does occur.

- Where the deployment allows, keep long-lived, pre-warmed connections (connection pooling/persistent sessions) for known trading clients so the SYN handshake is not repeated at the most time-critical moment of the trading day.

- Deploy application-layer/kernel-bypass techniques (e.g., DPDK or kernel-bypass NICs) for the most latency-critical paths, since even a well-tuned TCP stack retains inherent per-packet overhead that dedicated high-frequency systems often bypass entirely.